# M5StickC Plus Initial Setup and Testing

## Introduction

For the FYP I was given an M5StickC Plus v1.1 device to look into. The first thing I wanted to do was find out what the device actually is, what it can do and how I can program it.

The first test with the device was just to get it connected to my laptop and get a simple Hello World program running on the screen.

## What is the M5StickC Plus?

The M5StickC Plus is a small device that uses an ESP32 processor. It has a number of different sensors and features already built into it.

Some of the main features are:

- ESP32 processor

- LCD screen

- Accelerometer

- Gyroscope

- Wi-Fi

- Microphone

- Buzzer

- Buttons

- Built-in battery

- USB-C connection

- Grove connection for adding other devices or sensors

The accelerometer and gyroscope could be useful for the FYP, as they can be used to detect movement and rotation of the device.

When I first turned on the device, there was already a program installed on it that showed an axis moving around on the screen when the device was moved. This looked to be using the movement sensors inside the device.

## Connecting the M5StickC Plus

I connected the M5StickC Plus to my laptop using the USB-C and used the Arduino IDE to program it.

There were a few different COM ports showing in Arduino IDE, so I disconnected the M5Stick and checked which one disappeared. COM9 disappeared when the device was disconnected and came back when it was connected again.

## Setting up Arduino IDE

At first Arduino IDE did not have the ESP32 boards installed. I opened the Boards Manager and installed:

Esp32 by Espressif Systems Version 3.3.12

After installing this I was able to select:

Board: M5Stick-C

I then selected COM9 as the port.

The M5StickC Plus library also needed to be installed. There were a few different versions available including M5StickC, M5StickC Plus and M5StickC Plus2.

I installed M5StickCPlus because the device I am using is the M5StickC Plus v1.1.

## Hello World Test

Once everything was set up, I created a simple program to display Hello World on the screen.

#include <M5StickCPlus.h>

Void setup(){

M5.begin();

M5.Lcd.setRotation(1);

M5.Lcd.setTextSize(3);

M5.Lcd.setCursor(20,50);

M5.Lcd.println(“Hello World!”);

}

Void loop() {

}

The first line includes the M5StickC Plus library so I can use the different features of the device.

#include <M5StickCPlus.h>

Setup() runs when the device starts.

M5.begin();

This starts up the M5StickC Plus.

I then used:

M5.Lcd.setRotation(1);

To change the direction of the screen.

M5.Lcd.setTextSize(3);

Sets the size of the text and:

M5.Lcd.setCursor(20,50);

Sets where the text will appear on the screen.

Finally:

M5.Lcd.println(“Hello World!”);

Prints Hello World onto the screen.

There is nothing inside loop() because for this test I only needed the message to appear when the device started.

## Testing the Program

Before uploading the program, I used the Verify button in Arduino IDE to make sure the code could compile.

The code compiled successfully and used:

- 330,975 bytes of program storage, which was 10%

- 24,192 bytes of memory, which was 7%

I then uploaded the program to the M5StickC Plus using COM9.

The device restarted itself after the upload, and Hello World appeared on the screen.

This showed that the M5StickC Plus was connected correctly and I was able to write, compile, and upload a program onto it.

## What I Learned

From this first test, I now know what the M5StickC Plus is and some of the features that are built into it. I also know how to connect it to my laptop, set it up in Arduino IDE, and upload a basic program to it.

The device also has an accelerometer and gyroscope, which could possibly be useful later in the FYP for collecting movement data.

For now, the main aim was just to get the device working and successfully run a basic Hello World program on it.